# Section 1: Number systems and base conversions

1. Which number system uses only the digits 0 and 1?

A. Decimal B. Binary C. Octal D. Hexadecimal

Correct Answer: B. Binary

Explanation: The binary number system has base 2 and uses only 0 and 1. Computers use binary representation to process and store digital information.

2. What is the base (radix) of the octal number system?

A. 2 B. 8 C. 10 D. 16

Correct Answer: B. 8

Explanation: The octal system uses eight symbols, from 0 to 7. The hexadecimal system uses base 16, while decimal uses base 10.

3. Which of the following is a valid hexadecimal number?

A. 19G B. 128 C. 2AF D. 708

Correct Answer: C. 2AF

Explanation: Hexadecimal uses the digits 0–9 and letters A–F, representing values 10–15. The symbol G is not valid in hexadecimal, and 8 is not permitted in octal.

4. What is the decimal equivalent of binary 10102?

A. 8 B. 10 C. 12 D. 14

Correct Answer: B. 10

Explanation: Using positional weights, 10102=1(23)+0(22)+1(21)+0(20)=8+2=10.

5. What is the binary equivalent of decimal 1310?

A. 1011 B. 1100 C. 1101 D. 1110

Correct Answer: C. 1101

Explanation: 13=8+4+1=23+22+20. Therefore, the binary representation is 11012.

6. What is the decimal equivalent of octal 178?

A. 13 B. 14 C. 15 D. 17

Correct Answer: C. 15

Explanation: 178=1×8+7=1510. In positional notation, each digit is multiplied by the corresponding power of the base.

7. What is the decimal equivalent of hexadecimal A16?

A. 8 B. 9 C. 10 D. 11

Correct Answer: C. 10

Explanation: In hexadecimal, A represents decimal 10, B represents 11, and F represents 15.

8. Which is the correct binary representation of octal 258?

A. 10101 B. 10011 C. 11001 D. 10110

Correct Answer: A. 10101

Explanation: Convert each octal digit into three binary bits: 28=0102 and 58=1012. Thus, 258=0101012=101012.

9. What is the hexadecimal equivalent of binary 11112?

A. E B. F C. 10 D. 15

Correct Answer: B. F

Explanation: The four-bit binary value 11112 equals decimal 15, represented by F in hexadecimal. A group of four binary bits corresponds to one hexadecimal digit.

10. What is the binary equivalent of hexadecimal C16?

A. 1010 B. 1011 C. 1100 D. 1110

Correct Answer: C. 1100

Explanation: Hexadecimal C represents decimal 12. The binary representation of 12 is 11002.

11. What is the decimal equivalent of binary 1111112?

A. 31 B. 32 C. 63 D. 64

Correct Answer: C. 63

Explanation: 1111112=32+16+8+4+2+1=63. A binary number containing six 1s has a value of 26-1=63.

12. What is the decimal equivalent of hexadecimal 2F16?

A. 45 B. 46 C. 47 D. 48

Correct Answer: C. 47

Explanation: 2F16=2×16+15=32+15=4710.

13. What is the octal equivalent of decimal 6410?

A. 80 B. 100 C. 110 D. 64

Correct Answer: B. 100

Explanation: 64=1×82+0×8+0. Therefore, 6410=1008. A leading zero is not required.

14. What is the hexadecimal equivalent of decimal 25510?

A. FE B. FF C. F0 D. 100

Correct Answer: B. FF

Explanation: 255=15×16+15. Since hexadecimal F represents 15, the result is FF16.

15. Which number system is most directly associated with compactly representing groups of four binary bits?

A. Decimal B. Octal C. Hexadecimal D. Unary

Correct Answer: C. Hexadecimal

Explanation: Each hexadecimal digit represents exactly four binary bits. Octal digits represent groups of three bits, making hexadecimal especially convenient for displaying byte-oriented data.

16. What is the largest decimal value representable by an unsigned 4-bit binary number?

A. 8 B. 15 C. 16 D. 31

Correct Answer: B. 15

Explanation: An unsigned n-bit number can represent values from 0 to 2n-1. For four bits, the maximum is 24-1=15.

17. How many distinct values can be represented by an unsigned 8-bit number?

A. 8 B. 128 C. 255 D. 256

Correct Answer: D. 256

Explanation: Eight bits provide 28=256 distinct patterns. For unsigned representation, these correspond to decimal values 0 through 255.

18. Which of the following is NOT a valid octal number?

A. 127 B. 705 C. 816 D. 340

Correct Answer: C. 816

Explanation: Octal permits only digits 0 through 7. The digit 8 makes 8168 invalid, regardless of the values of its other digits.

19. What is the decimal equivalent of 10002?

A. 4 B. 6 C. 8 D. 10

Correct Answer: C. 8

Explanation: The leftmost 1 occupies the 23 position, so 10002=23=810.

20. What is the decimal equivalent of 1008?

A. 8 B. 16 C. 32 D. 64

Correct Answer: D. 64

Explanation: 1008=1×82=6410. The position of a digit determines the power of the base applied to it.

21. What is the decimal equivalent of 1016?

A. 10 B. 12 C. 16 D. 100

Correct Answer: C. 16

Explanation: The rightmost digit has weight 160, while the leftmost 1 has weight 161. Thus, 1016=1×16+0=1610.

22. What is the hexadecimal equivalent of decimal 3110?

A. 1E B. 1F C. 20 D. 2F

Correct Answer: B. 1F

Explanation: 31=1×16+15. The remainder 15 is represented by F, producing 1F16.

23. What is the octal equivalent of binary 1110002?

A. 56 B. 70 C. 71 D. 74

Correct Answer: B. 70

Explanation: Group the bits from the right into threes: 111 000. These groups represent octal digits 7 and 0, giving 708.

24. What is the binary equivalent of decimal 2010?

A. 10100 B. 10010 C. 11000 D. 10110

Correct Answer: A. 10100

Explanation: 20=16+4=24+22. Placing 1s at those positions gives 101002.

25. Which statement about positional number systems is correct?

A. Every number system uses base 10. B. The value of a digit depends on its position and the system's base. C. The digit 9 is valid in every number system. D. The value of a number is independent of its base.

Correct Answer: B. The value of a digit depends on its position and the system's base.

Explanation: In positional notation, each position represents a power of the base. For example, 102 equals 2, whereas 1010 equals 10.

26. What is the decimal equivalent of 1011012?

A. 43 B. 44 C. 45 D. 46

Correct Answer: C. 45

Explanation: 1011012=32+8+4+1=4510. Only positions containing 1 contribute to the decimal value.

27. Convert 7310 to binary.

A. 1001001 B. 1001010 C. 1010001 D. 1100101

Correct Answer: A. 1001001

Explanation: 73=64+8+1=26+23+20. Therefore, the binary representation is 10010012.

28. What is the decimal equivalent of 3458?

A. 221 B. 227 C. 229 D. 237

Correct Answer: B. 229

Explanation: 3458=3(82)+4(81)+5=192+32+5=229. Thus, the correct answer is C. 229.

29. What is the decimal equivalent of 1A316?

A. 403 B. 409 C. 419 D. 423

Correct Answer: C. 419

Explanation: 1A316=1(162)+10(16)+3=256+160+3=419.

30. Which binary number represents decimal 100?

A. 1100010 B. 1100100 C. 1100110 D. 1110000

Correct Answer: B. 1100100

Explanation: 100=64+32+4=26+25+22. The corresponding binary digits form 11001002.

31. What is the octal equivalent of 1111112?

A. 67 B. 76 C. 77 D. 107

Correct Answer: C. 77

Explanation: Group the binary digits into threes: 111 111. Each group represents octal 7, so the result is 778.

32. Convert 7B16 to decimal.

A. 121 B. 122 C. 123 D. 124

Correct Answer: C. 123

Explanation: B represents 11 in hexadecimal. Thus, 7B16=7(16)+11=112+11=123.

33. What is the hexadecimal equivalent of binary 101011112?

A. AF B. BA C. A7 D. BF

Correct Answer: A. AF

Explanation: Split the binary number into four-bit groups: 1010 1111. These represent hexadecimal A and F, respectively.

34. Which decimal number corresponds to 1101012?

A. 51 B. 52 C. 53 D. 55

Correct Answer: C. 53

Explanation: 1101012=32+16+4+1=5310.

35. What is the hexadecimal equivalent of decimal 128?

A. 7F B. 80 C. 88 D. 100

Correct Answer: B. 80

Explanation: 128=8×16+0. Hence, its hexadecimal representation is 8016.

36. Convert 101010102 to hexadecimal.

A. AA B. A5 C. 55 D. AC

Correct Answer: A. AA

Explanation: The two groups are 1010 and 1010, each representing A. Therefore, the answer is AA16.

37. What is the binary equivalent of octal 578?

A. 101111 B. 110111 C. 111101 D. 101101

Correct Answer: A. 101111

Explanation: Convert each octal digit into three bits: 5=101 and 7=111. Combining them gives 1011112.

38. What is the decimal equivalent of 1118?

A. 63 B. 71 C. 73 D. 81

Correct Answer: B. 73

Explanation: 1118=1(64)+1(8)+1=7310. Each position represents a power of 8.

39. Which is the correct hexadecimal representation of decimal 1000?

A. 3E8 B. 3D8 C. 3F8 D. 4E8

Correct Answer: A. 3E8

Explanation: 1000=3(162)+14(16)+8=768+224+8. Since E represents 14, the result is 3E816.

40. What is the decimal value of FF16?

A. 240 B. 254 C. 255 D. 256

Correct Answer: C. 255

Explanation: F represents 15, so FF16=15(16)+15=255. It is also the maximum unsigned value of an 8-bit byte.

41. Which of the following represents the same value as 100002?

A. 810 B. 1610 C. 208 D. Both B and C

Correct Answer: D. Both B and C

Explanation: 100002=24=1610. Octal 208 also equals 2×8=1610.

42. What is the decimal equivalent of 2D16?

A. 43 B. 44 C. 45 D. 46

Correct Answer: C. 45

Explanation: D represents 13. Therefore, 2D16=2(16)+13=45.

43. Which number system is commonly used to represent machine addresses compactly?

A. Unary B. Decimal only C. Hexadecimal D. Roman numerals

Correct Answer: C. Hexadecimal

Explanation: Hexadecimal provides a compact way to express binary data because each digit corresponds to four bits. Memory addresses and diagnostic values are commonly displayed in hexadecimal.

44. What is the largest single digit in the hexadecimal system?

A. 9 B. E C. F D. 16

Correct Answer: C. F

Explanation: F represents decimal 15 and is the largest hexadecimal digit. The value 16 is represented by 1016, not by a single digit.

45. What is the decimal equivalent of 1010102?

A. 40 B. 41 C. 42 D. 44

Correct Answer: C. 42

Explanation: 1010102=32+8+2=4210.

46. Convert decimal 50 to octal.

A. 52 B. 60 C. 62 D. 64

Correct Answer: C. 62

Explanation: 50=6(8)+2. Therefore, 5010=628.

47. What is the binary equivalent of F016?

A. 11110000 B. 11100000 C. 10110000 D. 11111100

Correct Answer: A. 11110000

Explanation: F corresponds to 1111 and 0 corresponds to 0000. Combining the two four-bit groups gives 111100002.

48. What is the decimal equivalent of 1234?

A. 21 B. 27 C. 31 D. 35

Correct Answer: B. 27

Explanation: 1234=1(42)+2(4)+3=16+8+3=27. Base 4 is valid here because all digits are less than 4.

49. Which of the following is a valid binary number?

A. 10201 B. 110101 C. 12011 D. 101201

Correct Answer: B. 110101

Explanation: Binary notation permits only 0 and 1. Any number containing 2 or another digit is invalid in base 2.

50. What is the octal equivalent of hexadecimal 1016?

A. 108 B. 168 C. 208 D. 218

Correct Answer: B. 208

Explanation: 1016=1610. Decimal 16 is represented as 208, since 2×8+0=16.

## Section 2: Binary addition and subtraction

51. What is the result of 1012+102?

A. 1102 B. 1112 C. 10002 D. 10112

Correct Answer: B. 1112

Explanation: 1012=5 and 102=2. Their sum is 7, which is 1112.

52. Calculate 11012+1012.

A. 100002 B. 100102 C. 101002 D. 111102

Correct Answer: A. 100102

Explanation: 11012=13 and 1012=5, so the sum is 18, or 100102. The correct result is 100102 (option B).

53. What is 1112+1112?

A. 11102 B. 11002 C. 10102 D. 10002

Correct Answer: A. 11102

Explanation: Each 1112 equals 7. Their sum is 14, represented as 11102.

54. What is the result of 10002-12?

A. 10012 B. 1112 C. 1102 D. 1012

Correct Answer: B. 1112

Explanation: 10002 represents 8. Subtracting 1 gives 7, whose binary representation is 1112.

55. Calculate 110102-1012.

A. 101012 B. 101112 C. 110012 D. 100112

Correct Answer: A. 101012

Explanation: 110102=26 and 1012=5. Their difference is 21, represented as 101012.

56. What is the binary sum of 11112 and 12?

A. 11112 B. 100002 C. 11102 D. 101002

Correct Answer: B. 100002

Explanation: 11112 is 15. Adding 1 produces 16, which requires five bits in ordinary unsigned binary notation.

57. Which rule applies when adding 1+1 in binary?

A. Sum 1, carry 0 B. Sum 0, carry 1 C. Sum 1, carry 1 D. Sum 0, carry 0

Correct Answer: B. Sum 0, carry 1

Explanation: Binary addition follows base 2. Since 1+1=210=102, the result bit is 0 and a carry of 1 is passed to the next position.

58. What is the result of 10112+1102?

A. 100012 B. 100002 C. 11112 D. 110012

Correct Answer: A. 100012

Explanation: 10112=11 and 1102=6. The sum is 17, represented as 100012.

59. What is 100002-12?

A. 11112 B. 10012 C. 10102 D. 11002

Correct Answer: A. 11112

Explanation: 100002=16. Subtracting 1 gives 15, which is 11112.

60. Which binary addition is correct?

A. 1012+112=10002 B. 1012+112=1112 C. 1012+112=10102 D. 1012+112=1102

Correct Answer: A. 1012+112=10002

Explanation: 1012=5 and 112=3. Their sum is 8, represented by 10002.

61. What is the result of 101102-1102?

A. 100002 B. 100102 C. 101002 D. 111002

Correct Answer: A. 100002

Explanation: 101102=22 and 1102=6. The difference is 16, or 100002.

62. What is the result of 111002+1002?

A. 111102 B. 1000002 C. 110002 D. 1010002

Correct Answer: B. 1000002

Explanation: 111002=28 and 1002=4. Their sum is 32, represented as 1000002.

63. What is the binary result of 1001012-10102?

A. 111012 B. 110112 C. 101112 D. 100012

Correct Answer: A. 110112

Explanation: 1001012=37 and 10102=10. The difference is 27, whose binary representation is 110112.

64. What is the binary result of 101012+101012?

A. 1010102 B. 1011012 C. 1100102 D. 1111112

Correct Answer: A. 1010102

Explanation: Adding a binary number to itself doubles its value. Here, 101012=21, so the result is 42, or 1010102.

65. Which statement about binary subtraction is correct?

A. Borrowing is never required. B. A borrow may be required when subtracting 1 from 0. C. The result can contain the digit 2. D. Binary subtraction always produces a positive result.

Correct Answer: B. A borrow may be required when subtracting 1 from 0.

Explanation: Binary subtraction uses borrowing when a position has insufficient value. A borrowed binary 1 represents 2 units in the next lower position, allowing 102-12=12.

66. What is the result of 111112+12?

A. 111102 B. 1000002 C. 1011112 D. 1111112

Correct Answer: B. 1000002

Explanation: 111112=31. Adding 1 produces 32, which is 1000002.

67. Calculate 1010002-11112.

A. 110012 B. 110102 C. 110112 D. 111012

Correct Answer: C. 110012

Explanation: 1010002=40 and 11112=15. Their difference is 25, represented by 110012.

68. What is the binary equivalent of decimal 15+17?

A. 111102 B. 1000002 C. 1000102 D. 1010002

Correct Answer: C. 1000002

Explanation: 15+17=32, and decimal 32 is 1000002. The correct result is option B.

69. What is the result of 11002+11002?

A. 110002 B. 111002 C. 101002 D. 100002

Correct Answer: A. 110002

Explanation: Each 11002 equals 12. Doubling 12 gives 24, which is 110002.

70. What is the result of 1000002-12?

A. 111102 B. 111112 C. 1000012 D. 110112

Correct Answer: B. 111112

Explanation: Decimal 32 minus 1 equals 31. In binary, 31 is represented by five consecutive 1s.

71. In binary addition, what is 1+1+1?

A. Sum 1, carry 1 B. Sum 0, carry 1 C. Sum 1, carry 0 D. Sum 0, carry 0

Correct Answer: A. Sum 1, carry 1

Explanation: The total is decimal 3, or 112. Therefore, the result bit is 1 and the carry bit is 1.

72. What is the binary result of 1011012-10012?

A. 1001002 B. 1001012 C. 1010002 D. 1101002

Correct Answer: A. 1001002

Explanation: 1011012=45 and 10012=9. The difference is 36, or 1001002.

73. What is the sum of 11102 and 10112?

A. 110012 B. 110102 C. 101012 D. 111012

Correct Answer: A. 110012

Explanation: 11102=14 and 10112=11. Their sum is 25, represented by 110012.

74. Which operation is represented by a left shift of one bit on an unsigned binary integer, when no significant bits are lost?

A. Division by 2 B. Multiplication by 2 C. Addition of 1 D. Subtraction of 1

Correct Answer: B. Multiplication by 2

Explanation: Shifting bits one position to the left appends a zero on the right and doubles the value, provided overflow does not occur. A right shift by one position corresponds to integer division by 2 for an unsigned integer.

75. Which statement best describes overflow in fixed-width unsigned binary arithmetic?

A. The result always becomes negative. B. The result gains unlimited extra bits. C. The mathematical result exceeds the representable range. D. The computer automatically changes the number system.

Correct Answer: C. The mathematical result exceeds the representable range.

Explanation: An unsigned n-bit value has a maximum of 2n-1. If an arithmetic result exceeds that limit, fixed-width arithmetic may discard the carry beyond the most significant bit.

## Section 3: Bits, bytes, words and character encoding

76. What is the smallest basic unit of digital data?

A. Byte B. Bit C. Word D. Kilobyte

Correct Answer: B. Bit

Explanation: A bit is a binary digit that can hold either 0 or 1. Bytes and words consist of multiple bits.

77. How many bits are there in one byte in standard modern computing?

A. 4 B. 8 C. 16 D. 32

Correct Answer: B. 8

Explanation: A byte consists of eight bits. It can represent 28=256 different bit patterns.

78. How many distinct patterns can an 8-bit byte represent?

A. 8 B. 64 C. 128 D. 256

Correct Answer: D. 256

Explanation: Each bit has two possible states. Thus, eight bits can represent 28=256 distinct patterns, from 00000000 to 11111111.

79. What does the term word generally mean in computer architecture?

A. A fixed unit of text only B. A processor's natural data unit, whose size depends on the architecture C. Always exactly 8 bits D. Always exactly 64 bits

Correct Answer: B. A processor's natural data unit, whose size depends on the architecture

Explanation: A word is the natural unit of data handled by a processor architecture. Word size varies; it may be 16, 32, 64 bits, or another architecture-defined size.

80. How many bits are in 4 bytes?

A. 16 B. 24 C. 32 D. 64

Correct Answer: C. 32

Explanation: One byte contains 8 bits, so 4×8=32 bits.

81. In the traditional binary-based convention, 1 KiB equals how many bytes?

A. 1000 B. 1024 C. 1048 D. 2048

Correct Answer: B. 1024

Explanation: One kibibyte (KiB) is 210=1024 bytes. Strictly, one kilobyte (kB) in the decimal SI convention is 1000 bytes.

82. How many bits are in 2 KiB?

A. 2048 B. 8192 C. 16,384 D. 32,768

Correct Answer: C. 16,384

Explanation: 2KiB=2×1024=2048 bytes. Multiplying by 8 gives 2048×8=16,384 bits.

83. Which encoding standard is traditionally associated with 7-bit character codes?

A. ASCII B. UTF-32 C. Unicode in all forms D. EBCDIC in all forms

Correct Answer: A. ASCII

Explanation: Standard ASCII uses 7 bits and defines 128 code points, from 0 through 127. Extended 8-bit character sets exist, but they are not standard 7-bit ASCII.

84. How many code points are defined by standard 7-bit ASCII?

A. 64 B. 128 C. 256 D. 512

Correct Answer: B. 128

Explanation: Seven bits produce 27=128 possible patterns. ASCII includes control characters and printable characters such as English letters and digits.

85. Which statement correctly describes Unicode?

A. It is limited to English letters. B. It is a character-encoding system that supports only 128 characters. C. It provides a universal character repertoire for many writing systems and symbols. D. It is another name for decimal notation.

Correct Answer: C. It provides a universal character repertoire for many writing systems and symbols.

Explanation: Unicode assigns code points to characters from many languages, including Bangla, as well as symbols and emoji. UTF-8, UTF-16 and UTF-32 are different ways of encoding Unicode code points.

86. Which encoding uses a variable number of bytes per Unicode code point, from one to four?

A. Standard ASCII B. UTF-8 C. UTF-32 only D. BCD

Correct Answer: B. UTF-8

Explanation: UTF-8 uses one to four bytes per Unicode code point. ASCII characters retain their original byte values in UTF-8, which helps maintain compatibility with ASCII-based systems.

87. Which encoding uses exactly four bytes per Unicode code point?

A. UTF-8 B. UTF-16 C. UTF-32 D. ASCII

Correct Answer: C. UTF-32

Explanation: UTF-32 represents each Unicode code point using a 32-bit code unit. UTF-8 uses one to four bytes, while UTF-16 uses one or two 16-bit code units.

88. Which statement about UTF-8 and ASCII is correct?

A. UTF-8 uses two bytes for every English letter. B. UTF-8 preserves ASCII byte values for ASCII characters. C. ASCII can encode every Unicode character. D. UTF-8 is a binary-to-decimal conversion method.

Correct Answer: B. UTF-8 preserves ASCII byte values for ASCII characters.

Explanation: ASCII characters are encoded in UTF-8 using the same values from 0 to 127. This makes ordinary English text compatible at the byte level with ASCII.

89. Which of the following is a Unicode code point notation?

A. U+0041 B. IP-0041 C. ASCII/0041 D. UTF/41

Correct Answer: A. U+0041

Explanation: Unicode code points are conventionally written as U+ followed by hexadecimal digits. For example, U+0041 represents the capital Latin letter A.

90. How many bytes does UTF-8 use to encode the ASCII character A?

A. 1 B. 2 C. 3 D. 4

Correct Answer: A. 1

Explanation: The letter A has ASCII value 65, or hexadecimal 41. UTF-8 encodes it as the single byte 41.

91. What is the main distinction between a character and its encoded representation?

A. A character and a byte are always identical. B. A character is an abstract textual unit, while its encoding maps it to bytes or code units. C. Every character requires exactly one bit. D. Characters cannot be represented numerically.

Correct Answer: B. A character is an abstract textual unit, while its encoding maps it to bytes or code units.

Explanation: A character is a conceptual unit of text; an encoding defines how it is represented digitally. A character may require different numbers of bytes under different encodings.

92. What is the hexadecimal ASCII value of the uppercase letter A?

A. 30 B. 41 C. 61 D. 7A

Correct Answer: B. 41

Explanation: Uppercase A has ASCII decimal value 65, equivalent to hexadecimal 41. Lowercase a is decimal 97, or hexadecimal 61.

93. What is the ASCII decimal code for the digit character 0?

A. 0 B. 30 C. 48 D. 64

Correct Answer: C. 48

Explanation: The character 0 has ASCII decimal value 48, or hexadecimal 30. The numeric value zero and the character representing the digit zero are not the same concept.

94. How many bits are needed to represent 32 distinct symbols using fixed-length binary codes?

A. 4 B. 5 C. 6 D. 8

Correct Answer: B. 5

Explanation: Five bits can represent 25=32 distinct patterns. Four bits provide only 24=16, which is insufficient.

95. What is the minimum number of bits needed to distinguish 100 different symbols using fixed-length codes?

A. 6 B. 7 C. 8 D. 10

Correct Answer: B. 7

Explanation: Six bits represent only 64 patterns, while seven bits represent 128. Therefore, seven bits are sufficient for 100 symbols.

96. Which statement about UTF-16 is correct?

A. It always uses exactly one byte per character. B. It uses 16-bit code units and may require two code units for a code point. C. It is identical to 7-bit ASCII. D. It cannot represent characters outside the Basic Multilingual Plane.

Correct Answer: B. It uses 16-bit code units and may require two code units for a code point.

Explanation: UTF-16 uses 16-bit code units. Code points in the supplementary planes are represented by a surrogate pair consisting of two code units.

97. Which character set is particularly important for storing Bangla text in modern digital systems?

A. ASCII alone B. Unicode C. Morse code alone D. BCD alone

Correct Answer: B. Unicode

Explanation: Unicode supports Bangla letters and other characters used in the language. UTF-8 is a common encoding for storing and transmitting Unicode text on the web.

98. If a file contains 500 bytes, how many bits does it contain?

A. 500 B. 1000 C. 4000 D. 8000

Correct Answer: C. 4000

Explanation: Each byte contains 8 bits. Therefore, 500×8=4000 bits.

99. Which unit is commonly used to measure data transmission speed?

A. Byte only B. Bits per second (bps) C. Pixels per inch D. Hertz only

Correct Answer: B. Bits per second (bps)

Explanation: Network transmission rates are commonly measured in bits per second, such as Mbps or Gbps. File sizes are commonly measured in bytes and their multiples.

100. A data file is 3 KiB in size. How many bytes does it contain?

A. 3000 B. 3072 C. 2048 D. 4096

Correct Answer: B. 3072

Explanation: One KiB equals 1024 bytes. Therefore, 3×1024=3072 bytes.

## Section 4: Boolean logic and basic logic gates

101. In Boolean algebra, what are the two possible values of a variable?

A. 0 and 1 B. 1 and 2 C. −1 and +1 only D. 0 through 9

Correct Answer: A. 0 and 1

Explanation: Boolean variables take two logical values, commonly represented as 0 (FALSE) and 1 (TRUE). They form the basis of digital logic and computer circuits.

102. What is the output of an AND gate when both inputs are 1?

A. 0 B. 1 C. Undefined D. Alternates between 0 and 1

Correct Answer: B. 1

Explanation: An AND gate produces 1 only when all its inputs are 1. For two inputs, the output is A∧B.

103. What is the output of an OR gate when its inputs are 0 and 1?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: B. 1

Explanation: An OR gate produces 1 when at least one input is 1. Its output is 0 only when all inputs are 0.

104. Which logic gate reverses its input?

A. AND B. OR C. NOT D. XOR

Correct Answer: C. NOT

Explanation: A NOT gate, also called an inverter, complements its input. An input of 1 produces 0, and an input of 0 produces 1.

105. What is the output of an AND gate for inputs 1 and 0?

A. 0 B. 1 C. 10 D. Undefined

Correct Answer: A. 0

Explanation: Both inputs must be 1 for an AND gate to produce 1. Since one input is 0, the output is 0.

106. Which gate produces 1 when its two inputs are different?

A. AND B. OR C. XNOR D. XOR

Correct Answer: D. XOR

Explanation: Exclusive OR (XOR) outputs 1 when the inputs differ and 0 when they are equal. It is widely used in arithmetic circuits and parity calculations.

107. What is the output of an XOR gate when both inputs are 1?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: A. 0

Explanation: XOR produces 1 only when its inputs differ. Because both inputs are 1, the output is 0.

108. Which gate produces the complement of an AND gate's output?

A. NOR B. NAND C. XOR D. XNOR

Correct Answer: B. NAND

Explanation: NAND means NOT-AND. It outputs 0 only when all inputs are 1; for two inputs, its output is A∧B.

109. Which gate produces 1 only when all its inputs are 0?

A. AND B. NAND C. OR D. NOR

Correct Answer: D. NOR

Explanation: NOR is the complement of OR. It produces 1 only when neither input is 1.

110. What is the output of a NAND gate for inputs 1 and 1?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: A. 0

Explanation: An AND gate produces 1 for inputs 1 and 1. NAND reverses that result, producing 0.

111. Which of the following gates is called a universal gate?

A. AND only B. OR only C. NAND D. XOR only

Correct Answer: C. NAND

Explanation: NAND is universal because any Boolean function can be implemented using NAND gates alone. NOR is also a universal gate.

112. What is the output of a NOR gate when both inputs are 0?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: B. 1

Explanation: OR produces 0 when both inputs are 0. NOR complements that output, producing 1.

113. Which expression represents an OR operation in Boolean algebra?

A. A⋅B B. A+B C. A D. A⊕B

Correct Answer: B. A+B

Explanation: Boolean OR is commonly written as A+B, while AND is written as A⋅B. The symbol ⊕ represents XOR.

114. What is the Boolean expression for an AND gate?

A. A+B B. A⋅B C. A D. A⊕B

Correct Answer: B. A⋅B

Explanation: The Boolean product A⋅B represents AND. Its output is 1 only if both A and B equal 1.

115. According to De Morgan's law, which expression is equivalent to A⋅B?

A. A⋅B B. A+B C. A+B D. A⋅B

Correct Answer: B. A+B

Explanation: De Morgan's law states that the complement of an AND operation equals the OR of the complemented inputs. This identity is important when converting between NAND, NOR and other gate implementations.

116. Simplify the Boolean expression A+0.

A. 0 B. 1 C. A D. A

Correct Answer: C. A

Explanation: In Boolean algebra, OR with 0 leaves the variable unchanged. This is known as the identity law.

117. Simplify A⋅1.

A. 0 B. 1 C. A D. A

Correct Answer: C. A

Explanation: AND with 1 leaves the other input unchanged. Thus, A⋅1=A.

118. What is the value of A+A?

A. 0 B. 1 C. A D. A

Correct Answer: B. 1

Explanation: A Boolean variable must be either 0 or 1. Therefore, either A or its complement is always 1, making their OR equal to 1.

119. What is the value of A⋅A?

A. 0 B. 1 C. A D. Depends on the value of A

Correct Answer: A. 0

Explanation: A and its complement can never both be 1. Their AND operation is therefore always 0.

120. Which statement correctly describes an XNOR gate?

A. It outputs 1 when inputs differ. B. It outputs 1 when inputs are equal. C. It always outputs 0. D. It is identical to a NOT gate with two inputs.

Correct Answer: B. It outputs 1 when inputs are equal.

Explanation: XNOR is the complement of XOR. For two inputs, it outputs 1 for 00 and 11, and 0 for 01 and 10.

121. If A=1 and B=0, what is the output of A∧B?

A. 0 B. 1 C. 10 D. Undefined

Correct Answer: A. 0

Explanation: The symbol ∧ represents logical AND. Since B is 0, both inputs are not true, and the output is 0.

122. If A=1 and B=0, what is the output of A⊕B?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: B. 1

Explanation: XOR returns 1 when its inputs differ. Since A is 1 and B is 0, the result is 1.

123. A circuit has output Y=A+B. Which gate does it represent?

A. AND B. NAND C. NOR D. XOR

Correct Answer: C. NOR

Explanation: The expression first ORs the inputs and then complements the result. This is the defining operation of a NOR gate.

124. Which gate can be used alone to construct an inverter?

A. NAND B. XOR only C. AND only D. OR only

Correct Answer: A. NAND

Explanation: Connect both inputs of a NAND gate to the same signal A. Its output becomes A⋅A=A, which is a NOT operation.

125. What is the output of Y=(A⋅B)+C when A=1, B=1, and C=0?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: B. 1

Explanation: First evaluate the AND operation: 1⋅1=1. Then calculate 1+0=1 using Boolean OR.

126. What is the output of Y=(A+B)⋅C when A=0, B=1, and C=0?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: A. 0

Explanation: First, A+B=0+1=1. Then 1⋅0=0, so the final output is 0.

127. Which logic gate is most directly used to test whether two binary bits differ?

A. AND B. OR C. XOR D. NOR

Correct Answer: C. XOR

Explanation: XOR outputs 1 when its two input bits are different. This property makes it useful in parity generation and binary addition.

128. In a half-adder circuit, which gates are commonly used to produce the sum and carry outputs?

A. OR for sum, NOT for carry B. XOR for sum, AND for carry C. NAND for sum, NOR for carry D. XNOR for sum, XOR for carry

Correct Answer: B. XOR for sum, AND for carry

Explanation: A half-adder adds two bits. The sum is A⊕B, while the carry is A⋅B.

129. What is the output of Y=A when A=0?

A. 0 B. 1 C. 2 D. Undefined

Correct Answer: B. 1

Explanation: The NOT operation reverses the logical value. Therefore, an input of 0 produces an output of 1.

130. Which of the following is a correct truth-table property of an AND gate?

A. Output 1 for every input combination B. Output 0 only when both inputs are 0 C. Output 1 only when both inputs are 1 D. Output 1 whenever the inputs differ

Correct Answer: C. Output 1 only when both inputs are 1

Explanation: For a two-input AND gate, only the input combination 11 produces an output of 1. The combinations 00, 01 and 10 all produce 0.

## Section 5: Data compression and error detection

131. What is the primary purpose of data compression?

A. To increase file size B. To reduce the amount of data needed to represent information C. To guarantee that data is encrypted D. To eliminate every possible transmission error

Correct Answer: B. To reduce the amount of data needed to represent information

Explanation: Compression reduces the storage space or transmission bandwidth needed for data. It does not automatically provide encryption or error correction.

132. Which type of compression allows the original data to be reconstructed exactly?

A. Lossy compression B. Lossless compression C. Analog compression only D. Destructive compression

Correct Answer: B. Lossless compression

Explanation: Lossless compression preserves all original information, allowing exact reconstruction. It is appropriate for text documents, program files and other data where changes are unacceptable.

133. Which of the following commonly uses lossy compression?

A. JPEG images B. Plain text compressed losslessly C. An exact archival copy of a database D. A lossless ZIP archive

Correct Answer: A. JPEG images

Explanation: JPEG commonly uses lossy compression to reduce image size by discarding some information. Lossless formats and methods preserve the original data exactly.

134. Which algorithm is associated with lossless compression using repeated sequences or runs of identical values?

A. Run-Length Encoding (RLE) B. RSA encryption C. Round-robin scheduling D. Frequency modulation

Correct Answer: A. Run-Length Encoding (RLE)

Explanation: RLE replaces repeated symbols with a symbol and a count. For example, AAAAA may be represented as A5, though the actual efficiency depends on the encoding format and data.

135. Which of the following is an example of lossless compression software or format?

A. ZIP B. JPEG with lossy settings C. MPEG video with lossy encoding D. Lossy MP3 encoding

Correct Answer: A. ZIP

Explanation: ZIP commonly uses lossless compression, preserving the original files when extracted. JPEG and many MP3 or MPEG encodings typically sacrifice some information to achieve smaller sizes.

136. What is the main difference between compression and encryption?

A. Both always reduce file size. B. Compression reduces representation size; encryption protects confidentiality. C. Encryption guarantees that files become smaller. D. Compression always requires a secret key.

Correct Answer: B. Compression reduces representation size; encryption protects confidentiality.

Explanation: Compression aims to represent data more efficiently. Encryption transforms data to prevent unauthorized reading; encryption does not inherently reduce file size.

137. Which technique is commonly used to detect accidental errors in transmitted data?

A. Parity checking B. Screen resolution C. File naming D. Font formatting

Correct Answer: A. Parity checking

Explanation: A parity bit is added to data so that the number of 1s is even or odd according to a chosen rule. It can detect certain errors but cannot reliably identify or correct every error.

138. In even parity, what is the value of the parity bit for data 1011?

A. 0 B. 1 C. 2 D. Cannot be determined

Correct Answer: B. 1

Explanation: The data 1011 contains three 1s, an odd count. Adding a parity bit of 1 produces four 1s, making the total count even.

139. In odd parity, what parity bit should be added to data 1100?

A. 0 B. 1 C. 2 D. No parity bit can be used

Correct Answer: B. 1

Explanation: 1100 contains two 1s, an even count. Adding 1 makes the total number of 1s odd, satisfying odd parity.

140. What is a major limitation of a single parity bit?

A. It cannot detect any single-bit error. B. It detects every possible multiple-bit error. C. It may fail to detect an even number of bit flips. D. It always corrects a corrupted bit automatically.

Correct Answer: C. It may fail to detect an even number of bit flips.

Explanation: A parity check detects any odd number of bit flips within the protected group, assuming the parity bit is included. An even number of flips can preserve the expected parity, so parity alone cannot guarantee detection of every error.

141. What does CRC stand for in data communication?

A. Central Routing Code B. Cyclic Redundancy Check C. Computer Retrieval Channel D. Common Register Control

Correct Answer: B. Cyclic Redundancy Check

Explanation: CRC is an error-detection technique based on polynomial arithmetic over binary data. It is widely used in network protocols and storage systems to detect accidental corruption.

142. Which statement about a checksum is correct?

A. It is always a form of encryption. B. It can help detect changes or corruption in data. C. It guarantees recovery of all corrupted data. D. It always compresses data.

Correct Answer: B. It can help detect changes or corruption in data.

Explanation: A checksum is a calculated value used to check whether data has changed or become corrupted. A simple checksum does not guarantee detection of every possible change and does not itself repair the data.

143. Which method is specifically designed to support correction of certain errors as well as detection?

A. Hamming code B. Run-Length Encoding C. JPEG compression D. ASCII encoding

Correct Answer: A. Hamming code

Explanation: Hamming codes add structured redundancy to data. A standard Hamming code can correct a single-bit error under its normal assumptions, while extended variants can also detect certain additional errors.

144. What is the purpose of redundancy in error-detection and error-correction systems?

A. To remove all data bits B. To add extra information that helps identify or correct errors C. To make every file smaller D. To convert binary data into decimal numbers only

Correct Answer: B. To add extra information that helps identify or correct errors.

Explanation: Redundant bits carry information beyond the original payload. They enable techniques such as parity checks, CRC and error-correcting codes to detect or correct corruption.

145. Which statement best distinguishes error detection from error correction?

A. Detection identifies evidence of an error; correction attempts to recover the intended data. B. Detection always repairs data, while correction only reports errors. C. Both terms mean compression. D. Error correction is impossible in digital systems.

Correct Answer: A. Detection identifies evidence of an error; correction attempts to recover the intended data.

Explanation: Error detection indicates that data may be corrupted. Error correction uses additional information or retransmission mechanisms to restore the correct data.

146. A data block contains 1000 bits. A compressed representation contains 700 bits. What is the percentage reduction in size?

A. 20% B. 25% C. 30% D. 70%

Correct Answer: C. 30%

Explanation: The reduction is 1000-700=300 bits. The percentage reduction is \((300/1000)\times100=30\%\).

147. A file is reduced from 8 MB to 2 MB by compression. What is the compression ratio, expressed as original size to compressed size?

A. 1:4 B. 2:1 C. 4:1 D. 8:1

Correct Answer: C. 4:1

Explanation: The ratio is 8:2, which simplifies to 4:1. The compressed file is one-quarter of the original size, representing a 75% reduction.

148. Which technique is most appropriate when exact recovery of a text file is required?

A. Lossy JPEG compression B. Lossless compression C. Lossy audio encoding D. Discarding redundant-looking characters without encoding them

Correct Answer: B. Lossless compression

Explanation: Lossless methods preserve the complete original information. Lossy methods may permanently discard information, so they are unsuitable when exact reconstruction is required.

149. Which of the following is a common application of CRC?

A. Detecting transmission errors in network frames B. Translating English into Bangla C. Increasing monitor brightness D. Assigning IP addresses to users

Correct Answer: A. Detecting transmission errors in network frames

Explanation: CRC values allow a receiver to check whether a frame's bits may have been altered during transmission. CRC is designed for error detection, not for encrypting messages.

150. Why is a cryptographic hash different from a simple error-detection checksum?

A. A cryptographic hash is designed to make it computationally difficult to find collisions and recover input information. B. A cryptographic hash always contains the original file. C. A checksum is always longer than the data it protects. D. A cryptographic hash is a type of binary addition only.

Correct Answer: A. A cryptographic hash is designed to make it computationally difficult to find collisions and recover input information.

Explanation: Cryptographic hashes are designed with security properties such as collision resistance and preimage resistance. Simple checksums primarily target accidental errors and generally do not provide comparable protection against deliberate tampering.

## Section 6: Mixed, application-based and challenging questions

151. What is the decimal equivalent of 10110112?

A. 89 B. 91 C. 93 D. 95

Correct Answer: B. 91

Explanation: 10110112=64+16+8+2+1=91. Converting binary to decimal requires summing the positional weights of all 1s.

152. What is the hexadecimal equivalent of decimal 4095?

A. FFF B. FFE C. 1000 D. EFF

Correct Answer: A. FFF

Explanation: 4095=163-1=4096-1. The largest three-digit hexadecimal value is FFF, equal to 15(256)+15(16)+15=4095.

153. A computer stores an unsigned integer in 12 bits. What is its maximum representable value?

A. 1023 B. 2047 C. 4095 D. 4096

Correct Answer: C. 4095

Explanation: The maximum unsigned value for n bits is 2n-1. For 12 bits, 212-1=4096-1=4095.

154. What is the minimum number of bits required to represent decimal 1000 as an unsigned binary integer?

A. 9 B. 10 C. 11 D. 12

Correct Answer: B. 10

Explanation: Nine bits can represent at most 29-1=511. Ten bits can represent values up to 210-1=1023, so 10 bits are sufficient.

155. Which is the correct binary representation of decimal 255?

A. 11111110 B. 11111111 C. 10000000 D. 10000001

Correct Answer: B. 11111111

Explanation: 255=28-1, so its 8-bit binary representation consists of eight 1s. This is the largest unsigned value stored in one byte.

156. Which hexadecimal value represents the 8-bit binary pattern 00001111?

A. 0F B. F0 C. FF D. 10

Correct Answer: A. 0F

Explanation: The binary pattern divides into 0000 and 1111, which represent hexadecimal 0 and F. The leading zero preserves the display as two hexadecimal digits.

157. What is the result of the bitwise AND operation between 1100 and 1010?

A. 1110 B. 1000 C. 0110 D. 0010

Correct Answer: B. 1000

Explanation: Bitwise AND produces 1 only when both corresponding bits are 1. Comparing the positions gives 1100 AND 1010 = 1000.

158. What is the result of the bitwise OR operation between 1100 and 1010?

A. 1000 B. 0110 C. 1110 D. 0010

Correct Answer: C. 1110

Explanation: Bitwise OR produces 1 whenever at least one corresponding bit is 1. Applying it to the two patterns produces 1110.

159. What is the result of the bitwise XOR operation between 1100 and 1010?

A. 0110 B. 1110 C. 1000 D. 0010

Correct Answer: A. 0110

Explanation: XOR produces 1 where corresponding bits differ. The first and third positions differ, giving 0110.

160. What is the result of the bitwise NOT operation on an 8-bit value 00001111?

A. 11110000 B. 00000000 C. 11111111 D. 00001110

Correct Answer: A. 11110000

Explanation: Bitwise NOT reverses every bit: 0 becomes 1 and 1 becomes 0. The fixed 8-bit width matters because NOT is applied to all eight positions.

161. In 8-bit unsigned arithmetic, what is the result of 250+10 if the result is stored using wraparound arithmetic?

A. 4 B. 255 C. 260 D. 0

Correct Answer: A. 4

Explanation: The mathematical sum is 260. Eight-bit unsigned arithmetic wraps modulo 256, so 260-256=4.

162. Which range can be represented by an 8-bit unsigned integer?

A. −128 to 127 B. 0 to 255 C. −255 to 255 D. 0 to 256

Correct Answer: B. 0 to 255

Explanation: An unsigned 8-bit integer represents 28=256 values, starting from 0. Its range is therefore 0 through 255 inclusive.

163. What is the range of an 8-bit signed integer using two's-complement representation?

A. 0 to 255 B. −127 to 127 C. −128 to 127 D. −128 to 128

Correct Answer: C. −128 to 127

Explanation: Two's-complement signed integers have a range from -2n-1 to 2n-1-1. For eight bits, this is −128 through 127.

164. What is the 8-bit two's-complement representation of decimal −1?

A. 00000001 B. 11111110 C. 11111111 D. 10000000

Correct Answer: C. 11111111

Explanation: To obtain the two's complement of +1, invert 00000001 to get 11111110, then add 1. The result is 11111111, representing −1 in 8-bit two's-complement form.

165. Which operation is performed when a binary value is shifted right by one bit using a logical right shift?

A. The value is always doubled. B. A zero enters the most significant position, and the other bits shift right. C. All bits become 1. D. The number is converted into hexadecimal.

Correct Answer: B. A zero enters the most significant position, and the other bits shift right.

Explanation: A logical right shift moves each bit one position to the right and fills the leftmost position with 0. For unsigned integers, it corresponds to division by 2 with the fractional part discarded.

166. Which statement about a binary fraction is correct?

A. Every decimal fraction has a terminating binary representation. B. Binary fractions use negative powers of 2 for positions after the binary point. C. Binary fractions cannot represent values below 1. D. Binary fractions use powers of 10 after the binary point.

Correct Answer: B. Binary fractions use negative powers of 2 for positions after the binary point.

Explanation: Positions after the binary point have weights 2-1,2-2,2-3, and so on. Some decimal fractions, such as 0.1, have repeating rather than terminating binary representations.

167. What is the decimal equivalent of binary 0.1012?

A. 0.5 B. 0.625 C. 0.75 D. 0.875

Correct Answer: B. 0.625

Explanation: 0.1012=1(2-1)+0(2-2)+1(2-3)=0.5+0.125=0.625.

168. What is the decimal equivalent of hexadecimal A.F16?

A. 10.15 B. 10.875 C. 10.9375 D. 15.625

Correct Answer: C. 10.9375

Explanation: A represents 10, while F represents 15. Therefore, A.F16=10+15/16=10.9375.

169. Which binary number is equivalent to decimal 0.5?

A. 0.012 B. 0.12 C. 0.112 D. 1.12

Correct Answer: B. 0.12

Explanation: The first fractional position in binary has weight 2-1=0.5. Thus, 0.12 equals decimal 0.5.

170. Which statement about floating-point representation is correct?

A. It can represent numbers with fractional parts and very large or very small magnitudes. B. It always represents every real number exactly. C. It uses only decimal digits internally. D. It eliminates rounding errors in all calculations.

Correct Answer: A. It can represent numbers with fractional parts and very large or very small magnitudes.

Explanation: Floating-point formats use components such as a significand and exponent to cover a wide range of values. Many real numbers cannot be represented exactly in a finite binary floating-point format, so rounding can occur.

171. Which coding scheme represents decimal digits using four-bit groups, with each decimal digit encoded separately?

A. ASCII B. BCD (Binary-Coded Decimal) C. UTF-8 D. Gray code

Correct Answer: B. BCD (Binary-Coded Decimal)

Explanation: In standard 8421 BCD, each decimal digit is represented separately by a four-bit pattern. For example, decimal 59 is encoded as 0101 1001, not as the pure binary representation of 59.

172. What is the BCD representation of decimal 27?

A. 11011 B. 0010 0111 C. 0001 1011 D. 0011 0101

Correct Answer: B. 0010 0111

Explanation: BCD encodes each digit independently: 2 becomes 0010 and 7 becomes 0111. Concatenating them gives 0010 0111.

173. Which statement correctly distinguishes pure binary from BCD?

A. Both always use four bits for every decimal digit. B. Pure binary represents the entire number in base 2, while BCD encodes each decimal digit separately. C. BCD can use only the digits 0 and 1 as decimal symbols. D. Pure binary cannot represent integers.

Correct Answer: B. Pure binary represents the entire number in base 2, while BCD encodes each decimal digit separately.

Explanation: Decimal 12 is 1100 in pure binary but 0001 0010 in standard BCD. BCD can be useful in decimal-oriented display and processing systems, though it may use more bits.

174. Which code changes only one bit between consecutive code values, a property useful in reducing ambiguity during transitions?

A. Gray code B. Standard ASCII C. UTF-32 D. BCD in all possible arrangements

Correct Answer: A. Gray code

Explanation: In reflected binary Gray code, adjacent code values differ in exactly one bit. This property is useful in applications such as rotary encoders and some position-sensing systems.

175. What is the 4-bit reflected Gray code equivalent of binary 1010?

A. 1111 B. 1110 C. 1011 D. 1101

Correct Answer: B. 1111

Explanation: To convert binary to Gray code, keep the most significant bit unchanged and XOR each pair of adjacent binary bits. For 1010, the result is 1111; therefore, the correct option is A.

176. Which is a correct statement about Unicode code points?

A. Every Unicode code point is represented by exactly one byte in every encoding. B. Unicode code points are abstract identifiers for characters or related text elements. C. Unicode supports only Latin scripts. D. Unicode is a compression algorithm.

Correct Answer: B. Unicode code points are abstract identifiers for characters or related text elements.

Explanation: A Unicode code point identifies a position in the Unicode codespace, conventionally written as U+.... Its encoded representation can occupy different numbers of bytes depending on the encoding, such as UTF-8 or UTF-16.

177. A message contains 1000 ASCII characters, all of which are standard ASCII characters. How many bytes are required to store their character data in UTF-8, excluding any file metadata?

A. 100 B. 1000 C. 2000 D. 8000

Correct Answer: B. 1000

Explanation: Each standard ASCII character occupies one byte in UTF-8. Therefore, 1000 such characters require 1000 bytes, excluding any additional formatting or metadata.

178. Which statement about a UTF-8 encoded Bangla character is generally correct?

A. It must always occupy exactly one byte. B. It may occupy multiple bytes. C. It is stored as a decimal integer without encoding. D. UTF-8 cannot encode Bangla characters.

Correct Answer: B. It may occupy multiple bytes.

Explanation: UTF-8 uses multiple bytes for code points outside its one-byte ASCII range. Bangla characters are generally encoded using multiple bytes in UTF-8.

179. Which data representation is most appropriate for a computer to process a Boolean TRUE/FALSE condition?

A. Two-state representation, commonly 1 and 0 B. Roman numerals only C. A mandatory 10-bit decimal string D. A graphical image in every case

Correct Answer: A. Two-state representation, commonly 1 and 0

Explanation: Digital logic commonly represents TRUE as 1 and FALSE as 0. The actual physical implementation may use voltage levels or other states, but the logical abstraction is binary.

180. Which statement about data redundancy is correct?

A. Redundancy is always harmful and must be eliminated. B. Redundancy can support error detection and correction. C. Redundancy always means encryption. D. Redundancy guarantees lossless compression will reduce every file's size.

Correct Answer: B. Redundancy can support error detection and correction.

Explanation: Additional bits or repeated information can help detect or recover from errors. However, redundancy can increase the amount of data stored or transmitted, and compression may not make every input file smaller.

181. A parity system uses even parity. A protected bit sequence has four 1s, and no parity bit has yet been added. What parity bit is required?

A. 0 B. 1 C. Either 0 or 1 D. 4

Correct Answer: A. 0

Explanation: The sequence already contains an even number of 1s. Adding a parity bit of 0 preserves even parity.

182. A receiver uses even parity and receives a block with an odd number of 1s, including the parity bit. What can it conclude?

A. The block definitely contains no error. B. A parity error has been detected. C. The exact erroneous bit is always known. D. The block has automatically been corrected.

Correct Answer: B. A parity error has been detected.

Explanation: An odd count violates the expected even-parity rule, indicating a detected error under the scheme's assumptions. The parity bit alone does not identify the exact erroneous bit or correct it.

183. Which technique generally provides stronger accidental-error detection than a simple single parity bit for network frames?

A. CRC B. Font encoding C. Lossy compression D. Decimal conversion

Correct Answer: A. CRC

Explanation: CRC uses a polynomial-based calculation to detect many error patterns that a single parity bit can miss. Its effectiveness depends on the chosen CRC polynomial and the error pattern.

184. Which of the following is NOT a purpose of error-correcting codes?

A. Detecting certain corrupted bit patterns B. Correcting certain transmission errors C. Adding redundancy to the transmitted data D. Guaranteeing confidentiality against unauthorized readers

Correct Answer: D. Guaranteeing confidentiality against unauthorized readers

Explanation: Error-correcting codes provide reliability, not confidentiality. Encryption is used to protect the secrecy of data, while error-correcting codes help detect or repair corruption.

185. Which situation is best suited to lossless compression?

A. Reducing the size of a document that must be restored exactly B. Discarding subtle details in a photograph to reduce its size C. Reducing audio quality intentionally D. Removing selected database records permanently

Correct Answer: A. Reducing the size of a document that must be restored exactly

Explanation: Lossless compression permits exact reconstruction of the original data. Lossy compression is often used for images, audio or video when some information loss is acceptable.

186. A 12 MB file is compressed to 9 MB. What is the percentage reduction in size?

A. 15% B. 20% C. 25% D. 33.33%

Correct Answer: C. 25%

Explanation: The reduction is 12-9=3 MB. The percentage reduction is \((3/12)\times100=25\%\).

187. If a compressed file is 40% of its original size, what percentage of the original size has been saved?

A. 40% B. 50% C. 60% D. 140%

Correct Answer: C. 60%

Explanation: The compressed file occupies 40% of the original size. The saving is \(100\%-40\%=60\%\).

188. Which statement about a compression ratio of 5:1, defined as original size to compressed size, is correct?

A. The compressed file is five times larger than the original. B. The compressed file is one-fifth of the original size. C. The file size has increased by 500%. D. The original and compressed files are equal in size.

Correct Answer: B. The compressed file is one-fifth of the original size.

Explanation: A 5:1 ratio means the original data size is five times the compressed size. If the original is 100 MB, the compressed size is 20 MB, assuming that ratio applies to the file.

189. Which technique is commonly used in lossless data compression by assigning shorter codes to more frequent symbols?

A. Huffman coding B. XOR encryption C. Parity checking D. Two's-complement conversion

Correct Answer: A. Huffman coding

Explanation: Huffman coding constructs variable-length codes, typically giving shorter codes to more frequent symbols and longer codes to less frequent ones. It is a lossless coding method often used as part of compression systems.

190. What is a key property of a prefix-free code such as a properly constructed Huffman code?

A. No codeword is the prefix of another codeword. B. Every codeword must have the same length. C. Every symbol must be encoded as exactly one bit. D. The code can only represent decimal digits.

Correct Answer: A. No codeword is the prefix of another codeword.

Explanation: A prefix-free code permits unambiguous decoding without needing separators between codewords. Huffman coding is a common example of a prefix-free code.

191. Which statement about compression of already compressed data is generally correct?

A. It always halves the file size. B. It may provide little benefit or even increase the size. C. It always converts lossy data into lossless data. D. It guarantees that all redundancy disappears.

Correct Answer: B. It may provide little benefit or even increase the size.

Explanation: Compressed files often contain less exploitable redundancy than their originals. Recompressing them can yield negligible savings, while headers or additional coding overhead may increase the size.

192. Which component is typically responsible for representing digital data using binary states in electronic circuits?

A. Logic circuits and storage elements B. Paper margins C. Printer toner D. Monitor stand

Correct Answer: A. Logic circuits and storage elements

Explanation: Digital electronic systems use physical states to represent logical 0s and 1s. Logic gates process these values, while registers and memory elements store them.

193. What is the decimal value of the 8-bit binary number 01111111 when interpreted as a two's-complement signed integer?

A. −127 B. 127 C. 128 D. 255

Correct Answer: B. 127

Explanation: The most significant bit is 0, so the number is non-negative. Its value is 64+32+16+8+4+2+1=127.

194. What is the decimal value of the 8-bit two's-complement number 10000000?

A. −128 B. −127 C. 128 D. 0

Correct Answer: A. −128

Explanation: In 8-bit two's-complement representation, the minimum value is -27=-128. This bit pattern is the minimum representable signed integer, not positive 128.

195. Which operation can be used to determine whether two binary values are equal, using an XOR result followed by a NOT operation across the compared bits?

A. XNOR-based comparison B. Lossy compression C. Parity generation only D. Decimal addition

Correct Answer: A. XNOR-based comparison

Explanation: XNOR outputs 1 when corresponding bits match. Combining bitwise comparisons can help determine whether two binary words are equal.

196. Which is the correct decimal value of 1101.012?

A. 13.125 B. 13.25 C. 13.5 D. 14.25

Correct Answer: B. 13.25

Explanation: The integer part 11012 equals 13. The fractional part .012 equals 0(2-1)+1(2-2)=0.25, giving 13.25.

197. Which hexadecimal number is equivalent to binary 0011 1100 1010?

A. 3CA B. 3AC C. 2CA D. 3EA

Correct Answer: A. 3CA

Explanation: Group the binary bits into sets of four: 0011, 1100, 1010. These correspond to hexadecimal 3, C and A, giving 3CA.

198. A system uses 16-bit words. How many bytes are in one word?

A. 1 B. 2 C. 4 D. 8

Correct Answer: B. 2

Explanation: One byte contains 8 bits. A 16-bit word therefore contains 16/8=2 bytes.

199. Which statement about binary representation is correct?

A. Binary can represent only whole numbers. B. Binary can represent integers, fractions, characters and logical values through appropriate encoding schemes. C. Binary is used only for storing photographs. D. Every character must have a unique single-bit representation.

Correct Answer: B. Binary can represent integers, fractions, characters and logical values through appropriate encoding schemes.

Explanation: Binary is the underlying representation used by digital computers for many kinds of data. The interpretation depends on the encoding or data format, such as integer formats, floating-point standards, ASCII or Unicode.

200. A digital system receives a binary data block and must detect accidental corruption, while also supporting confidentiality. Which combination best addresses both requirements?

A. ASCII and decimal conversion B. Compression and character encoding alone C. An appropriate error-detection mechanism and encryption D. Binary addition and file renaming

Correct Answer: C. An appropriate error-detection mechanism and encryption

Explanation: An error-detection mechanism, such as CRC, can identify many forms of accidental corruption. Encryption protects confidentiality; if protection against deliberate modification is also required, an authenticated encryption scheme or an appropriate message authentication mechanism is preferable to CRC alone.